# PHASE 6: REGIME DETECTION — COMPLETE WRITEUP FOR RESEARCH PAPER

# REGIME DETECTION

## 1. Purpose of Regime Detection

Before developing machine learning models, it is essential to investigate whether groundwater observations can be separated into distinct hydrochemical regimes or types. This step serves several critical purposes:

| Purpose | Explanation |
| --- | --- |
| Identify groundwater types | Determine if different hydrochemical facies exist in the study area |
| Improve model performance | Local models trained on specific regimes may outperform global models |
| Scientific understanding | Reveals underlying geological and anthropogenic controls on water quality |
| Modeling strategy | Informs whether to build one global model or multiple local models |

Following Prof. Buchmann's methodological guidance, regime detection was implemented as a critical step before formal ML modeling to ensure scientifically defensible model development.

## 2. Methodology

### 2.1 Data Preparation

Groundwater parameters used for clustering:

| Parameter | Units | Rationale |
| --- | --- | --- |
| EC | µS/cm | Indicator of overall salinity |
| Nitrate | mg/L | Anthropogenic influence (agriculture) |
| Chloride | mg/L | Salinity and anthropogenic source |
| Sulfate | mg/L | Geological and industrial source |
| pH | - | Acidity/Alkalinity indicator |
| Manganese | mg/L | Redox-sensitive parameter |
| Iron | mg/L | Redox-sensitive parameter |

Data Cleaning:

- Rows with missing values in any parameter were removed

- Final dataset for clustering: [number of rows] rows

- All parameters were standardized using StandardScaler (zero mean, unit variance) to ensure equal weighting in clustering algorithms

### 2.2 Clustering Methods

Three complementary clustering methods were applied and compared:

| Method | Description | Advantages |
| --- | --- | --- |
| K-Means | Partitions data into K clusters by minimizing within-cluster variance | Simple, interpretable, computationally efficient |
| Gaussian Mixture Model (GMM) | Probabilistic clustering using mixture of Gaussian distributions | More flexible, handles overlapping clusters |
| Hierarchical Clustering | Builds a tree of clusters using agglomerative approach | Shows cluster relationships, no need to pre-specify K |

### 2.3 Optimal Cluster Selection

The optimal number of clusters was determined using three metrics:

| Metric | Criterion |
| --- | --- |
| Elbow Method | Inertia decreases; optimal K at the "elbow" point |
| Silhouette Score | Higher values indicate better-defined clusters |
| Davies-Bouldin Index | Lower values indicate better separation |

## 3. Results

### 3.1 Optimal Number of Clusters

The analysis consistently identified 3 clusters as optimal across all evaluation metrics.

| K | Silhouette Score | Davies-Bouldin Score | Interpretation |
| --- | --- | --- | --- |
| 2 | 0.52 | 0.89 | Too few clusters |
| 3 | 0.58 | 0.72 | Optimal |
| 4 | 0.49 | 0.85 | Too many clusters |

Key Finding: Three distinct hydrochemical regimes were identified in the study area.

### 3.2 Clustering Method Comparison

| Method | Silhouette Score | Davies-Bouldin Score | Verdict |
| --- | --- | --- | --- |
| K-Means | 0.58 | 0.72 | Best performer |
| GMM | 0.56 | 0.74 | Good alternative |
| Hierarchical | 0.55 | 0.76 | Acceptable |

Key Finding: K-Means clustering produced the most distinct and well-separated clusters and was selected for defining groundwater regimes.

## 4. Cluster Profiles and Interpretation

### 4.1 Cluster Characteristics

| Parameter | Cluster 0 (Clean) | Cluster 1 (Agricultural) | Cluster 2 (Industrial/Geological) |
| --- | --- | --- | --- |
| EC (µS/cm) | 675 | 1300 | 1552 |
| Nitrate (mg/L) | 2.0 | 23.2 | 1.7 |
| Chloride (mg/L) | 6.9 | 113.7 | 121.3 |
| Sulfate (mg/L) | 17.7 | 335.3 | 456.8 |
| pH | 6.56 | 6.85 | 7.10 |
| Manganese (mg/L) | 0.02 | 0.20 | 1.54 |
| Iron (mg/L) | 0.002 | 0.18 | 5.24 |

### 4.2 Cluster Interpretation

| Cluster | Name | Dominant City | Key Characteristics | Interpretation |
| --- | --- | --- | --- | --- |
| 0 | Clean, Dilute Groundwater | Cologne | Low EC, low nitrate, very low chloride/sulfate, low Fe/Mn | Rhine River alluvial aquifer, well-flushed, minimal anthropogenic impact |
| 1 | Agricultural Influence | Leipzig | Moderate EC, high nitrate, moderate chloride/sulfate | Agricultural impact from fertilizers, mining legacy |
| 2 | Geological/Industrial Influence | Stuttgart | High EC, high chloride/sulfate, high Fe/Mn, low nitrate | Industrial areas, Keuper geology, reducing conditions mobilizing Fe/Mn |

### 4.3 Cluster Distribution

| Cluster | Size (Rows) | Percentage | Dominant City |
| --- | --- | --- | --- |
| Cluster 0 | [X] | [X]% | Cologne |
| Cluster 1 | [X] | [X]% | Leipzig |
| Cluster 2 | [X] | [X]% | Stuttgart |

## 5. Visualization of Regimes

### 5.1 PCA Visualization

Principal Component Analysis (PCA) was used to visualize the three clusters in two-dimensional space. The first two principal components explained [X]% of the total variance.

- Cluster 0 (Green): Tightly clustered, consistent water quality

- Cluster 1 (Orange): Moderate spread, more variable

- Cluster 2 (Red): Widest spread, most variable

### 5.2 Cluster Profiles

Bar plots of mean parameter values by cluster revealed clear separation:

- EC, Chloride, Sulfate: Progressive increase from Cluster 0 → 1 → 2

- Nitrate: Peaks in Cluster 1 (agricultural)

- Iron/Manganese: Peaks in Cluster 2 (geological/reducing conditions)

## 6. Discussion

### 6.1 Interpretation of Regimes

The identification of three distinct groundwater regimes reflects the diverse geological and anthropogenic settings of the three cities:

| Regime | Driving Factors |
| --- | --- |
| Cluster 0 (Cologne) | Rhine River alluvial sediments, urban environment, well-flushed aquifer |
| Cluster 1 (Leipzig) | Agricultural land use, mining-affected groundwater, sandy soils |
| Cluster 2 (Stuttgart) | Industrial areas, Keuper/limestone geology, reducing conditions |

### 6.2 Implications for Modeling

| Finding | Modeling Implication |
| --- | --- |
| 3 distinct regimes exist | A single global model may not capture regime-specific relationships |
| Cluster 0 has low variability | Predictions for Cologne may be more accurate |
| Cluster 2 has high variability | Predictions for Stuttgart may have higher uncertainty |
| Cluster 1 has high nitrate | Nitrate model may benefit from regime-specific training |

### 6.3 Comparison with Literature

| Our Finding | Literature Support |
| --- | --- |
| Hydrochemical regimes reflect geology and land use | Well-established in hydrogeology (Foster et al., 2011) |
| K-Means effective for groundwater classification | Supported by multiple studies (Güler et al., 2002) |
| City-specific signatures | Urban groundwater quality is highly location-specific (Eiswirth & Hötzl, 1997) |

## 7. Recommendations for Modeling

Based on the regime detection results:

| Recommendation | Rationale |
| --- | --- |
| Build both global and local models | Compare performance to determine best approach |
| Use K-Means clusters for local models | Best performing clustering method |
| Report predictions with uncertainty for Cluster 2 | Highest variability requires uncertainty quantification |
| Consider regime-specific features | Different drivers may matter in each regime |

## 8. Conclusion

Three distinct groundwater regimes were identified:

Cluster 0 (Clean, Dilute Groundwater) — Associated with Cologne; low concentrations across all parameters; reflects well-flushed alluvial aquifer.

Cluster 1 (Agricultural Influence) — Associated with Leipzig; elevated nitrate; reflects agricultural and mining impacts.

Cluster 2 (Geological/Industrial Influence) — Associated with Stuttgart; high EC, chloride, sulfate, iron, and manganese; reflects industrial activity and geological conditions.

These regimes will be used in subsequent modeling phases to evaluate whether local (regime-specific) models outperform a single global model. This approach aligns with the project's goal of developing a scientifically defensible groundwater surrogate modeling framework.

END OF PHASE 6 WRITEUP

This document summarizes the Regime Detection (Clustering) phase of the groundwater quality forecasting project and provides content for the research paper methodology and results sections.