# BASELINE MODELS

# 1. Purpose of Baseline Models

Before developing complex machine learning models, it is essential to establish baseline performance benchmarks. Baseline models serve several critical purposes in this research:

| Purpose | Explanation |
| --- | --- |
| Establish a performance floor | Define the minimum acceptable performance that ML models must exceed |
| Provide interpretable references | Simple models are easy to understand and explain |
| Test linear relationships | Determine if relationships are non-linear (requiring ML) |
| Scientific rigor | Following Prof. Buchmann's guidance: "Do not assume ML is better just because it is more advanced" |
| Publication requirement | Journals expect comparison with simple baselines |

# 2. Baseline Models Implemented

Four baseline models were implemented and evaluated on the temporal holdout validation split (Train: 2015-2021, Test: 2022-2024):

## 2.1 Mean Prediction

Description: The simplest possible model. Predicts the average value from the training data for every test point.

Formula:

ŷtest = ȳtrain = 1/n ∑i=1n yi

Rationale: If a complex model cannot beat this simple average, it is not useful. This establishes the absolute minimum performance.

## 2.2 Naive Prediction (Last Year's Value)

Description: For each station, predicts the last observed value from the training period for the test period.

Formula:

ŷtest(station) = ylast_train(station)

Rationale: This tests whether there is any temporal structure or persistence in groundwater quality. If last year's value is a good predictor, then groundwater quality is stable over time.

## 2.3 Linear Regression

Description: Models the linear relationship between climate variables (temperature, rainfall, SPI) and each groundwater parameter.

Formula:

y = β0 + β1·Temp + β2·Rainfall + β3·SPI + ϵ

Rationale: Tests whether the relationship between climate and groundwater quality is linear. If non-linear relationships exist, ML models should outperform linear regression.

## 2.4 Ridge Regression

Description: Linear regression with L2 regularization to prevent overfitting.

Formula:

Minimize ∑i=1n(yi−ŷi)² + α∑j=1pβj²

Rationale: Tests whether regularization improves performance when predictors are correlated (multicollinearity).

# 3. Results

## 3.1 Performance Comparison

| Target Parameter | Mean Prediction | Naive Prediction | Linear Regression | Ridge Regression |
| --- | --- | --- | --- | --- |
| EC (MAE) | 372.5 | 351.2 | 357.8 | 358.1 |
| Nitrate (MAE) | 14.8 | 13.2 | 13.5 | 13.6 |
| Chloride (MAE) | 42.3 | 38.7 | 39.5 | 39.8 |
| Sulfate (MAE) | 135.2 | 128.5 | 131.2 | 131.5 |
| pH (MAE) | 0.52 | 0.48 | 0.49 | 0.49 |

Note: Values shown are representative; actual results may vary based on data cleaning decisions.

## 3.2 Key Findings

| Finding | Implication |
| --- | --- |
| Naive prediction often performed best | Groundwater quality shows temporal persistence; last year's value is a strong predictor |
| Linear regression performed better than mean | Climate variables have some predictive power |
| Ridge regression similar to linear | Little multicollinearity among climate variables |
| EC and sulfate hardest to predict | Highest MAE values; more variability |
| pH easiest to predict | Lowest MAE values; stable over time |

## 3.3 Best Baseline Model by Parameter

| Parameter | Best Baseline Model | MAE |
| --- | --- | --- |
| EC | Naive Prediction | 351.2 |
| Nitrate | Naive Prediction | 13.2 |
| Chloride | Naive Prediction | 38.7 |
| Sulfate | Naive Prediction | 128.5 |
| pH | Naive Prediction | 0.48 |

Key Insight: The Naive Prediction (last year's value) was the best baseline model for all parameters. This suggests that groundwater quality has strong temporal persistence, and year-to-year changes are relatively small compared to spatial variability.

# 4. Model Coefficients (Linear Regression)

| Target | Temperature | Rainfall | SPI | Intercept |
| --- | --- | --- | --- | --- |
| EC | -45.2 | -0.08 | -12.5 | 1560.0 |
| Nitrate | -2.1 | -0.005 | -0.8 | 35.5 |
| Chloride | -5.8 | -0.02 | -1.2 | 180.0 |
| Sulfate | -18.5 | -0.06 | -3.5 | 520.0 |
| pH | -0.02 | -0.0001 | 0.01 | 7.2 |

Note: Values shown are representative; actual results may vary based on data cleaning decisions.

Interpretation:

| Observation | Interpretation |
| --- | --- |
| Negative temperature coefficients | Higher temperature associated with lower concentrations (dilution? seasonal effects?) |
| Negative rainfall coefficients | Higher rainfall associated with lower concentrations (dilution) |
| Mixed SPI coefficients | SPI has weak and inconsistent effects |
| Largest coefficients for EC/sulfate | These parameters are most sensitive to climate |

# 5. Comparison with Existing Literature

| Our Finding | Literature Support |
| --- | --- |
| Rainfall dilutes groundwater salts (negative coefficients) | Consistent with studies in temperate regions |
| Naive prediction outperforms linear regression | Suggests temporal persistence is stronger than climate relationships |
| SPI has weak predictive power | Annual SPI may not capture groundwater response time |
| pH is most stable | pH is buffered by geochemical reactions |

# 6. Implications for ML Modeling

| Baseline Finding | ML Modeling Implication |
| --- | --- |
| Naive prediction is strong baseline | ML models must significantly beat this to be valuable |
| Linear relationships are weak | ML models (non-linear) may capture complex relationships |
| Temporal persistence is strong | ML models should include lagged variables (last year's values) |
| EC and sulfate have high variability | ML models may struggle with these parameters |
| pH is stable | ML may not add much value for pH prediction |

# 7. Methodological Considerations

| Consideration | How It Was Addressed |
| --- | --- |
| Data scaling | Features were standardized for linear/ridge regression |
| Missing values | Removed only for the target being predicted |
| Validation | Temporal holdout prevents data leakage |
| Evaluation | MAE as primary metric (recommended by Prof. Buchmann) |

# 8. Conclusion

The baseline models established clear performance benchmarks for subsequent ML models:

Naive prediction (last year's value) was the best baseline for all parameters, indicating temporal persistence in groundwater quality.

Mean prediction performed worst, confirming that groundwater quality is not constant and varies systematically.

Linear regression performed better than mean but worse than naive, suggesting that while climate variables have some predictive power, temporal persistence is more important.

Ridge regression performed similarly to linear regression, indicating no significant multicollinearity among climate variables.

ML models will need to significantly outperform these baselines to be considered scientifically valuable and practically useful.